华夏云天试车台调试/标定/培训发动机要求

Engine Requirement of Skyaero test cell commission/correlation

- 试车台测试用（Sweeper/Shakedown）发动机要求：

Engine requirement

- 合作方式：租赁

- Cooperation mode：Lease

- 发动机型号：CF34-8C5A3

- Engine mode：CF34-8C5A3

- 时间节点要求:

Schedule:

- 需要2025年02月21日到达华夏云天（预计租期1个月，需求数量1台）

Ship to Skyaero on 21Feb2024（Estimate rental period: 1 month，rental QTY: 1 EA）

- 发动机测试预计使用20个循环；

Estimate engine Sweeper/Shakedown：20cycles;

- 性能要求：

Requirement of Performance

- 发动机能各系统功能正常，需提供在翼试车报告(或同等文件)给我方评估；

The engine can be functioned normally in all systems, and the on wing test report(or equivalent) should be provided for our evaluation;

- 发动机各模块内部无不可用损伤，需提供下发后整机孔探报告给我方评估；

No not serviceable damage can be found inside each module of the engine, and the whole engine borescope inspection report needs to be provided for our evaluation;

- 发动机能达到起飞功率并保持5分钟；

The engine can be reached to take-off power and maintain it for 5 minutes;

- 时受件-发动机寿命件最低剩余循环数大于100；

LLP-The minimum remaining cycle – at least 100;

- EGT裕度-大于10 °C

EGT margin- at least 10 °C

备注：需要确认租赁结束后发动机是否需要重新取证。

Note: It is necessary to confirm whether the engine needs to be re-certified after the end of the lease.

- 标定发动机要求:

Engine requirement

- 合作方式：租赁

- Cooperation mode：Lease

- 发动机型号：CF34-8C5A3

- Engine mode：CF34-8C5A3

- 时间节点要求:

Schedule:

- CF34-8C ,2025年2月21日前运抵美国GE标准台（预计租期2.5个月，需求数量1台），3月21前运抵华夏云天；

CF34-8C ships to GE before 21Feb2025 (Estimate Rental period: 2.5 months, Rental QTY: 1 EA), ships to Skyaero on 21Mar2025;

- 发动机标定预计使用15个循环；

Estimate engine correlation：15 cycles;

- 性能要求（以下英文为GE要求）：

Requirement of Performance

- Engine FADEC must be compatible with the engine models that will be applicable to the correlation.

发动机FADEC必须与标定发动机型号匹配

- More than 1,000 flight cycles and 1,000 flight hours in service since new or last shop visit with performance restoration and more than 1,000 cycles remaining of the limiting LLP.

自新发或最后一次进厂性能恢复后超过1000个飞行循环和1000个飞行小时，且时寿件剩余可用循环数超过1000 cycle。

- In-flight trend data and last Test Cell Performance Data must be provided to GE AVIATION for review.

飞行趋势数据和最近的试车台性能数据必须提供给GE航空以供审查。

- Trend data must show stability for EGT, Core Speed and Fuel Flow.

趋势数据必须能体现EGT，核心机转速和燃料流量的稳定性。

- No abnormal or unexplained shifts in the trends.

趋势中没有异常或无法解释的变化。

- EGT variation (flight to flight) must be within Engine Program Experience.

每个航班之间的EGT变化必须在发动机计划经验范围内。

- Thrust margin must allow engine to run up to the maximum corrected fan speed in the test without exceeding the physical fan speed redlines up to the expected range of ambient conditions.

在环境条件的预期范围内，推力裕度必须允许发动机在测试中达到最大修正风扇转速，而不超过物理风扇转速红线。

- EGT margin must allow engine to reach the maximum corrected fan speed specified in the test procedure without exceeding the Maximum Continuous EGT limit up to the expected range of ambient conditions.

在环境条件的预期范围内，EGT裕度必须允许发动机达到测试程序中规定的最大修正风扇转速，而不超过最大连续EGT极限。

- Core Speed margin must allow engine to run up to the maximum corrected fan speed specified in the test without exceeding the physical core speed redlines up to the expected range of ambient conditions.

在环境条件的预期范围内，核心机转速裕度必须允许发动机达到测试中指定的最大校正风扇转速，而不超过物理核心机转速红线。

备注：需要确认租赁结束后发动机是否需要重新取证。

Note: It is necessary to confirm whether the engine needs to be re-certified after the end of the lease.

- 培训用发动机要求

Train Engine requirement

- 合作方式：采购

- Cooperation mode：Purchase

- 发动机型号：CF34-8C5A3

- Engine mode：CF34-8C5A3

- 时间节点要求：2024年10月份发动机到我司工厂（需求数量：1台）

Schedule：The engine will be delivered to factory in October 2024（Purchase Qty：1 EA）

- 性能要求：

Requirement of Performance

- 最好包含全套的QEC及附件

It is best to include a full set of QEC and accessories

- 发动机状态：不可用

Engine status: Unavailable